Socket Programming Assignment: Web Server

For this assignment, you use the basics of socket programming for TCP connections in Python. You will

- create a socket

- bind it to a specific address and port

- send and receive a HTTP messages

- use the HTTP header formats as we did with telnet

Task

Develop a web server that handles one HTTP request at a time. Your web server should accept and parse the HTTP request message, get the requested file from the server’s file system (you must create this file as well, see below), create an HTTP response message consisting of the requested file preceded by appropriate message header lines, and then send the response directly to the client.

If the requested file is not present in the server, your server will send an HTTP “404 Not Found” message back to the client.

Running the Server

- Put a simple HTML file (e.g., HelloWorld.html) in the same directory that your web server is in.

- Run the server program.

- Determine the IP address of your host that is running the server. From the same or a different host, open a browser and provide the corresponding URL (IP+port+file).

  - For example:  http://131.229.102.177:6789/HelloWorld.html

  - ‘HelloWorld.html’ is the name of the file you placed in your server directory.

Note the use of the port number after the colon. You need to replace this port number with whatever port you have used in your server code. The browser should then display the contents of HelloWorld.html.

  - What happens if you forget the port number? Why?

- Next try to get a file that is not present at the server. You should get the “404 Not Found” message that your server sends to your web browser (client).

What to Hand in

Hand in the complete, well commented, server code along with the screen shots of your client browser, verifying that you actually receive the contents of the HTML file from the server.