### Problem 13

In the problem statement, the “/24” indicates that the right hand byte is available for you to assign. There is more than one possible assignment. One option is:

223.1.17.00/26 (or simply 223.1.17.0/26) provides for addresses 0 – 63 for subnet 1, with, therefore, 4 more than the minimum required.

223.1.17.128/25 specifies the right hand byte as 1xxx xxxx

223.1.17.192/28 specifies 1100 xxxx for subnet 3. Note that though these 16 addresses could be part of subnet 2 address space, with the rule of longest prefix matching, these 16 addresses (providing for the minimum number of 12 required) will be properly routed to subnet 3.

### Problem 14

Referring to the table on page 318, we can assign the addresses as shown below. Note that although it appears that link 2 would also get the addresses properly destined for link 1, with the rule of longest prefix matching, this assignment works.

Destination Address 	Link Interface

200.23.16/21		0

200.23.24/24		1

200.23.24/21		2

otherwise 		3

### Problem 27

b)  Dijkstra’s algorithm table for node u

| Step | N’ | D(x), p(x) | D(t),p(t) | D(v),p(v) | D(w),p(w) | D(y),p(y) | D(z),p(z) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | u | ∞ | 2,u | 3,u | 3,u | ∞ | ∞ |
|  | ut | ∞ | 2,u | 3,u | 3,u | 9,t | ∞ |
|  | utv | 6,v | 2,u | 3,u | 3,u | 9,t | ∞ |
|  | utvw | 6,v | 2,u | 3,u | 3,u | 9,t | ∞ |
|  | utvwx | 6,v | 2,u | 3,u | 3,u | 9,t | 14,x |
|  | utvwxy | 6,v | 2,u | 3,u | 3,u | 9,t | 14,x |
|  | utvwxyz | 6,v | 2,u | 3,u | 3,u | 9,t | 14,x |

Forwarding table for node u:

| Destination | Next Hop | Link |
| --- | --- | --- |
| t | t | (u, t) |
| v | v | (u, v) |
| w | w | (u, w) |
| x | v | (u, v) |
| y | t | (u, v) |
| z | x | (u, x) |

Note that you can write either the ‘next-hop’ or the ‘Link’ (outgoing link) in the forwarding table. You do not need both!

### Problem 28 – Distance Tables for node z

Note that you only need to include rows for the directly connected neighbors in the distance table, as those are the only nodes from which z receives distance vectors.

Cost to

u	v	x	y	z

v	∞	∞	∞	∞	∞

From	x	∞	∞	∞	∞	∞

z	∞	6	2	∞	0

Cost to

u	v	x	y	z

v	1	0	3	∞	6

From	x	∞	3	0	3	2

z	7	5	2	5	0

Cost to

u	v	x	y	z

v	1	0	3	3	5

From	x	4	3	0	3	2

z	6	5	2	5	0

Cost to

u	v	x	y	z

v	1	0	3	3	5

From	x	4	3	0	3	2

z	6	5	2	5	0

Forwarding table for node z:

| Destination | Next Hop | Link |
| --- | --- | --- |
| u | x | (z, x) |
| v | x | (z, x) |
| x | x | (z, x) |
| y | x | (z, x) |

Note again that you can write either the ‘next-hop’ or the ‘Link’ (outgoing link) in the forwarding table. You do not need both.